Document Folder

51-Satellite Snow Visualization Web Tool

- Do you have the specific algorithm from your grad student that you mentioned available? The FDL processing script written in python 3 with open source software is provided in the file “FDL_LDS_MOD10A1F.txt”  There is a Word document “FDL Processing Script Information-2026-09-02 which provides a rationale for decisions about how the algorithm was written (or how it jumps into the data to find either the FDL or LDS for a pixel and where the algorithm goes from the initial jump-in point).

Do we have a plan or budget for long term hosting/storage?

I have a plan for long/term hosting of the project through UI’s RCDS (Research Computing and Data Services).  The Idaho Water Resources Institute (IWRRI) that this project is associated with has made arrangements with them to assist and I believe hosting.  I have yet to get details from them.

What NDSI threshold did the paper use? Typically in scientific literature, a threshold around 40 is used, and we used a value in that range in our 2019 paper.  In subsequent research we found that we got clearer, more robust results (in FDLs and the PCA model) if we used an NDSI value of 10 as the threshold for snow (NDSI>10), and no-snow (NDSI<10).  We found out that using the NDSI threshold of 10 to work up through the PCA model, that we could then use that model to do cloud-gap-filling (CGF) across a range of NDSI values up to about 55 and the results were as good or better than using using the same threshold value to generate the PCA as that desired for conducting CGF.  This is very beneficial in running to the model operation in that it means the most computationally intensive part of the process (generating the FDLs) can be done once for regardless of what NDSI threshold the user wants to use to CGF.  We believe it also will allow a course reconstitution of the actual NDSI daily images (i.e., as NDSI values) in addition to producing CGF images that are binary snow/no-snow.  This can be done by performing CGF on an image using a whole set or incremental range of NDSI thresholds (although just a single threshold for processing up through the PCA) and combining these back into a single NDSI image based on the collection of binary CGF images produced. The multiple threshold values chosen impose the coarseness of the resulting CGF image.  That is, if one generates CGF images for a given day with threshold values of 10, 11, 12, 13,…, 55, and combines these images together one will get a finer increment of NDSI image than if one generates the CGF images for thresholds of 10, 15, 20, 25, …55.  The former gives an NDSI resolution of 1, whereas the latter’s resolution is 5.

As an additional note, the FDL production doesn’t require working with smaller areas (e.g., limited watershed scale). Instead, FDLs and LDSs can be produced for large areas, e.g., the entire mountainous western US.  These FDLs/LDSs can then be saved perpetually for processing into PCA models at smaller watershed scales.  This is beneficial since this component of the work is the most computationally intensive, as noted above.

Can we have the Upper Snake watershed boundary the paper used? With it, our pixel count can match the paper's 41,503.

Do you still have the 2000–2016 FDL rasters or the PC1 raster? With those we could compare pixel by pixel instead of by eye.

In the FDL search, did cloud and other flag values count as "not land"?  If by “not land” you mean “unknown” then yes.  In other words, pixels are treated as though they don’t count as either snow or no-snow and one must move to a different day’s image incrementally one day at a time until an actual threshold value (0-100), either later in time for FDL production, or earlier in time for LDS.

- When did the search stop, and what happened to pixels that never showed land? The FDL processing script written in python 3 with open source software is provided in the file “FDL_LDS_MOD10A1F.txt”  There is a Word document “FDL Processing Script Information-2026-09-02.docx” which provides a rationale for decisions about how the algorithm was written (or how it jumps into the data to find either the FDL or LDS for a pixel and where the algorithm goes from the initial jump-in point).

- Did the PCA use the covariance or the correlation matrix?

- The code, or the former student's version with a note on which version produced the paper.

- The watershed boundary file or mask it used.

- The settings it ran with: threshold, start dates, and any other run-time, or environment values.

- The output FDL and PC1 rasters, if they're still around.

In short, given the starter code, algorithm, and dataset used for verification will allow us to start ironing out the details and begin building an MVP.

I (Dr. Qualls) just received a response from my former student (Dr. Woodruff) about many of these questions, some of which are additional information about questions I have responded to above.  For expedience I am copy/pasting his responses below, rather than trying to interlace them with my responses above.

Responses from Dr. Woodruff:

- Do you have the specific algorithm from your former grad student that you mentioned available?

  - Yes all of the scripts can be made available

- Do we have a plan or budget for long term hosting/storage?

  - Dr. Qualls to respond.

- What NDSI threshold did the paper use?

  - We download the NDSI snow cover in the manuscript we used MOD10A1 https://nsidc.org/data/mod10a1/versions/61 NDSI snow cover - (refer to the data table)  which varies between 0-100. We use >=10 as snow covered and <10 as snow free. The script I shared with Russell Qualls calculated the FDL and LDS using the MOD10A1F dataset https://nsidc.org/data/mod10a1f/versions/61. MOD10A1 is cloud containing (that is what will be used for cloud removal but FDL and LDS can be efficiently calculated with the cloud gap filled (cloud free dataset). You will notice that the MOD10A1F data contains the original cloud containing data as one of the hdf layers.

- Can we have the Upper Snake watershed boundary the paper used? With it, our pixel count can match the paper's 41,503.

  - I'll package and share the shapefile by Monday of next week.

- Do you still have the 2000–2016 FDL rasters or the PC1 raster? With those we could compare pixel by pixel instead of by eye.

  - Yes, I'll share with Russell Qualls via onedrive or would the google drive be preferred? Monday of next week.

- In the FDL search, did cloud and other flag values count as "not land"? When did the search stop, and what happened to pixels that never showed land?

  - There are a number of decisions that must be considered which are all explicitly handled in the FDL processing script. Values refer directly to the data value look up tables for MOD10A1 and MOD10A1F.

- Did the PCA use the covariance or the correlation matrix?

  - We implemented the scikit learn PCA algorithm - actually a single value decomposition. We inpt directly the FDL matrices.

- The code, or the former student's version with a note on which version produced the paper.

  - It would be best to move to the newest version of the body of work. The  paper in question used a NDSI snow cover threshold of 40, which has since changed. Instead the newest versions of the scripts in open source python 3 libraries will be shared. I am still in the process of updating two stages FDL -> PCA -> PCA to cloud removal although these are simpler tasks. To be completed and shared by no later than October 16.

- The watershed boundary file or mask it used.

  - Monday of next week.

- The settings it ran with: threshold, start dates, and any other run-time, or environment values.

  - The settings that are of the greatest importance are in the FDL extraction and are all contained in the script already shared with Russell Qualls.

- The output FDL and PC1 rasters, if they're still around.

  - Monday of next week.